Yes. I’d make them built-in PROC/FUNC intrinsics or library routines, not brand-new statement syntax. That matches existing Action! style much better: the Atari library already exposes graphics/game features as routines like Graphics, SetColor, Plot, DrawTo, Sound, SndRst, Paddle, and Stick, instead of inventing lots of one-off language statements.

For the C64, I’d also design the API around the real hardware model. VIC-II gives you 8 hardware sprites with per-sprite position and color, plus shared bitmapped control for enable, X-MSB, expansion, priority, multicolour, and collisions; the sprite pointers normally live at 2040–2047. SID gives you 3 voices with per-voice frequency, pulse width, control/waveform, and ADSR, plus shared filter routing and master volume.

The big design choice: do not model SID after Atari POKEY distortion too closely. Atari Action!’s Sound routine is built around voice, pitch, distortion, and volume, but SID is much more naturally expressed as waveform + ADSR + filter + optional pulse width. Keep Sound only as a compatibility wrapper; make the real C64 API be Sid*.

## Sprite set I’d add

PROC SpriteOn(BYTE n)
PROC SpriteOff(BYTE n)
PROC SpritePos(BYTE n,CARD x,BYTE y)
PROC SpritePtr(BYTE n,BYTE ptr)
PROC SpriteData(BYTE n,CARD addr)
PROC SpriteColor(BYTE n,BYTE col)
PROC SpriteMC(BYTE n,BYTE on)
PROC SpriteXExp(BYTE n,BYTE on)
PROC SpriteYExp(BYTE n,BYTE on)
PROC SpritePrio(BYTE n,BYTE behind)
PROC SetSpriteMC(BYTE c0,BYTE c1)

BYTE FUNC SpriteHit()
BYTE FUNC SpriteHitBg()

Why these names work:

- SpritePos should hide the ugly shared X-MSB register and just accept x as a CARD.

- SpriteData is the friendly version; SpritePtr is the raw VIC pointer version.

- SpriteMC, SpriteXExp, SpriteYExp, and SpritePrio map directly to common VIC sprite controls without exposing bit twiddling.

- SpriteHit and SpriteHitBg are natural as functions because the collision registers are read-back values and the collision latches clear on read.

## SID set I’d add

PROC SidFreq(BYTE v,CARD freq)
PROC SidPulse(BYTE v,CARD width)
PROC SidWave(BYTE v,BYTE wave)
PROC SidAD(BYTE v,BYTE ad)
PROC SidSR(BYTE v,BYTE sr)
PROC SidOn(BYTE v)
PROC SidOff(BYTE v)

PROC SidVol(BYTE vol)
PROC SidCutoff(CARD cutoff)
PROC SidRes(BYTE res)
PROC SidMode(BYTE mode)
PROC SidRoute(BYTE mask)
PROC SidRst()

BYTE FUNC SidOsc3()
BYTE FUNC SidEnv3()

Why this set makes sense:

- SidFreq, SidPulse, SidWave, SidAD, and SidSR line up with the actual per-voice SID registers.

- SidOn/SidOff are friendlier than forcing everyone to manage the gate bit themselves.

- SidVol, SidCutoff, SidRes, SidMode, and SidRoute reflect the shared filter/volume section cleanly.

- SidOsc3 and SidEnv3 expose the useful readable SID state without forcing raw register reads.

## Compatibility layer I’d keep

PROC Sound(BYTE voice,pitch,distortion,volume)
PROC SndRst()

But I would document Sound as simple legacy sugar, not the preferred SID interface. Internally, it could map a small distortion table onto SID waveforms and use a fixed ADSR.

## Constants worth shipping

SID_TRI
SID_SAW
SID_PULSE
SID_NOISE

SID_LOW
SID_BAND
SID_HIGH

SPR_FRONT
SPR_BACK

## The one thing I would not add

I would not add lots of raw register keywords like D015, D01C, D418, etc. Power users already have the Action!-style Peek/Poke escape hatch, so your first-class API should stay semantic and compact.

A pretty good first cut would be just this subset:

SpriteOn SpriteOff SpritePos SpriteData SpriteColor
SidFreq SidWave SidAD SidSR SidOn SidOff SidVol
Sound SndRst

That gives you enough to write real games quickly, while leaving raster IRQs, multiplexing, and music-driver integration for a second layer.

Next step would be to sketch these as actual Action! declarations and show how each maps onto VIC-II and SID registers.

Top of Form

Bottom of Form